# SPI Overview

The SPI module provides a standardized serial interface to transfer data between MSPM0 devices and other external devices using SPI protocols (such as a Sensors, Memory, ADCs, or DACs).

### Purpose of the Peripheral

The SPI module acts as a controller or peripheral interface for synchronous serial communication with peripheral devices and other controllers. The transmit and receive paths are buffered with internal, independent FIFO memories allowing up to 4 entries with 16-bit width. A DMA interface is also provided to allow the data exchange with the transmit and receive FIFOs.

### Features

The SPI modules have the following features:

- Configurable as a controller or a peripheral

- Programmable clock bit rate and prescaler

- Separate transmit (TX) and receive (RX) first-in first-out buffers (FIFOs)

- Programmable data frame size from 4-bits to 16-bits (Controller Mode)

- Programmable data frame size from 7-bits to 16-bits (Peripheral Mode)

- Interrupts for transmit and receive FIFOs, overrun and timeout interrupts, and DMA done

- Programmable SPI mode support Motorola SPI or Texas Instruments format

- Direct memory access controller interface (DMA):

  - Separate channels for transmit and receive

  - Transfer complete interrupt

### Functional Block Diagram

IIDX

IMASK

RIS

MIS

ISET

ICLR

SPI Control

BUSCLK

MFCLK LFCLK

Clock Control

Functional Clock

Event Interface

Event Interrupts DMA RX Trigger

DMA TX Trigger

Interrupt Control

TxFIFO 4 x 16

DMA_DONE_RX

DMA_DONE_TX

Clock Prescaler

Data

Transmit/ Receive Logic

Control/Status

RxFIFO 4 x 16

RXDATA

TXDATA

CLKDIV

CLKSEL

CLKCTL

SCLK PICO POCI CS0 CS1 CS2

CS3 / CD

CTL0

CTL1

IFLS

STAT

## Figure 26-1. SPI Functional Block Diagram

### External Connections and Signal Descriptions

Figure 26-2 and Table 26-1 show an overview of the pin functions for different operation modes of the SPI module.

Peripheral SCLK

POCI PICO CS

Controller

SCLK POCI PICO CS0

Peripheral SCLK

POCI

PICO

Controller

SCLK POCI PICO

3 Wire Connec on	4 Wire Connec on

Peripheral SCLK

POCI

PICO CS CD

Controller

SCLK POCI PICO

CS0 CS3/CD

4 Wire Connec on with CD

4 Wire Connec on with 4 Peripherals

Controller

SCLK POCI PICO

CS0 CS1

CS2 CS3

Peripheral

SCLK POCI PICO

CS

Peripheral

SCLK POCI PICO

CS

Peripheral

SCLK POCI PICO

CS

Peripheral SCLK

POCI

PICO CS

Figure 26-2. External Connections for Different SPI Configurations

Table 26-1. Pin Function Overview

| Standard SPI | Feature |
| --- | --- |
| SCLK | SPI clock Controller mode: SCLK is an output Peripheral mode: SCLK is an input |
| PICO | Controller out, peripheral in Controller mode: PICO is the data output line Peripheral mode: PICO is the data input line |
| POCI | Controller in, peripheral out Controller mode: POCI is the data input line Peripheral mode: POCI is the data output line |
| CS0 | Chip select signal 0, used in 4-pin mode |
| CS1 | Chip select signal 1, used in 4-pin mode |
| CS2 | Chip select signal 2, used in 4-pin mode |
| CS3/CD | Chip select signal 3 or Command/Data, used in 4-pin mode |

For UNICOMM, to meet timing requirements in controller mode, chip select and clock should be looped back from IOs. This compensates for the IO delays within the SoC. At the device's IOMUX level, CS and SCLK INENA bits will need to be set.

# SPI Operation

### Clock Control

The SPI internal functional clock is selected and divided from the clock sourced to this module.

- Use SPIx.CLKSEL register to select the source of the SPI functional clock.

  - BUSCLK: the current bus clock is selected as the source for SPI. The current bus clock depends on power domain. If the SPI instance is in power domain 1 (PD1) please refer to MCLK, if the SPI instance is in power domain 0 (PD0) refer to ULPCLK.

  - MFCLK: MFCLK is selected as the source for SPI, refer to MFCLK.

  - LFCLK: LFCLK is selected as the source for SPI, refer to LFCLK

- Use SPIx.CLKDIV register to select the divide ratio of the SPI function clock. Options are from divide by 1 to 8.

The SPI module must be enabled before being configured for use by using the CTL0.EN bit in SPIx.PWREN register (see peripheral power enable). When the SPI is set up, or the configuration needs to be changed, after enabling power, the SPI peripheral must be configured to the desired configuration and then enabled via CTL0.EN bit to avoid unpredictable behavior during the updates or for the first data received or transmitted afterward.

The maximum SPI frequency supported in the controller and peripheral mode depends on the device clock option and IO option. Please refer to specific device data sheet spec for more information.

### General Architecture

      - Chip Select and Command Handling

        - Chip Select Control

To configure the SPI to be in controller mode by setting the CTL1.CP bit to 1, and to be in peripheral mode by clearing the CTL1.CP bit.

The CTL0.CSSEL bit selects which connected peripheral is addressed by the up to 4 CS signals. The bits are controlled by the SPI module in controller or target/peripheral mode. The selected signal is controlled during the transfers.

The chip select signal needs to be provided by the controller in four-wire mode and the chip select polarity can be inverted by configuring the PINCM.CSx.INV register.

In peripheral mode, the clock is provided by the controller and used by the peripheral to capture the data. The peripheral has the option to operate in 3-wire or 4-wire mode. 4-wire mode only accepts data transfers if the CS is activated.

The CTL0.CSCLR bit controls whether the transmit/receive shift register is automatically cleared when the chip select (CS) signal becomes inactive:

          - CTL0.CSCLR = 0: The bit counter state is preserved when CS deactivates the peripheral.

          - CTL0.CSCLR = 1: The bit counter is reset to zero when CS deactivates the peripheral.

When using CTL0.CSCLR = 1, observe these timing constraints to ensure proper synchronization:

          - CS Inactive Period: Maintain CS in the inactive state for at least two functional clock cycles before reasserting it.

          - CS Setup Time: Allow at least 2 SPI functional clock cycles between CS activation and the first clock edge.

These timing requirements help the peripheral resynchronize with the controller after clock-line disturbances or glitches, or during system initialization. This feature applies only when operating in peripheral mode.

## Note

The CSCLR function requires the CS disable pulse to be longer than 2 SPI function clock cycles to proper detect and clear the bit counter in the SPI. The CS lead time (CS active to the first bit clock edge) also needs to be at least 2 SPI function clock cycles.

        - Command Data Control

When using the Motorola frame format, the CDMODE bit can be set to use the CS3/CD line as signal to distinguish between Command and Data information. This is often used for LCD or data storage devices.

          - CD level low: command function

          - CD level high: data function

Command Function: To send out COMMAND, load CDMODE with 0xF and populate the transmit FIFO with the COMMAND data to be sent; the module will drive CS3/CD low and transmit the COMMAND data on the TX line.

Data Function with CDMODE as counter: After the COMMAND data is sent, load CDMODE with the number of data bytes to be transmitted and populate the transmit FIFO with the data to be sent. CS3/CD line will

be driven high & data will be transmitted on the TX line; CDMODE will be decremented by 1 after each transmission. CS3/CD line remains high after the counter reaches '0'.

Alternatively, the DATA function can also be implemented in MANUAL mode, by writing CDMODE with a value of '0' and then populating the transmit FIFO. This will pull CS3/CD high and transmit populated in the transmit FIFO.

## Note

1. The CDMODE bit is available only when the CDENABLE configuration bit is set; while the CDENABLE bit can be updated only when the SPI IP is disabled, the CDMODE bit can be updated at any time. 2. The CDMODE bit is not write-locked when counting down; therefore, a write to the CDMODE bit will restart the counter with the new value. 3. Before writing a new value into the CDMODE bit, ensure that the FIFO is empty.

      - Data Format

The control bit CTL1.MSB defines the direction of data input and output: most-significant-bit (MSB) or least- significant-bit (LSB) first. If the parity is enabled, the parity bits are always received as the last bit.

With the control register bits CTL0.DSS, the bit length per transfer will be defined between 4-16 bits for Controller mode and 7-16 bits for Peripheral mode.

A transfer will be triggered by writing to the TX buffer register. The data write needs to have at least the number of bits of the transfer. For example, if only a byte is written to the TX buffer, but the length of the transfer is > 8, the missing bits will be filled with 0s. On the receive path, the received data will be moved to the RXFIFO or RX buffer after the number of bits defined in the CTL0.DSS register has been received.

The RX and TX buffer shall be accessed with at least the bits covering one transfer.

- 4-8 bits : byte access (Peripheral mode: 7-8 Bits)

- 9-16 bits : 16 bit access

Clock polarity (CTL0.SPO) is used to control the clock polarity when data is not being transferred, and it is only used in theMotorola SPI frame mode.

- 0h = peripheral produces a steady state LOW value on the CLKOUT pin when data is not being transferred.

- 1h = peripheral produces a steady state HIGH value on the CLKOUT pin when data is not being transferred.

The Clock phase (CTL0.SPH) bit selects the clock edge that captures data and enables the data to change state. It has the greatest impact on the first bit transmitted, either by permitting or not permitting a clock transition before the first data capture edge. Please refer to the Motorola SPI frame mode section to check the diagrams. (Add line here)

- 0h = Data is captured on the first clock edge transition.

- 1h = Data is captured on the second clock edge transition.

The SPI can be configured to work in Peripheral mode with CTL1.CP bit = 0. In Peripheral mode, the clock is provided by the controller and is available to the peripheral on the CLK pins, which must be configured as inputs. The CS input signal selects/enables the peripheral's data receive path in 4-wire mode.

The SPI can be configured to work in Controller mode with CTL1.CP bit = 1. In Controller mode, the clock needs to be generated by selecting a clock source using the clock select bits. It also needs to control the CS signal depending on the selected protocol.

When setting the CTL1.PEN bit, the last bit will be used as parity to evaluate the integrity of the previous bits. The CTL1.PES bit selects the parity mode (even or odd). When detecting a fault, the interrupt flag RIS.PER is set to mark the data as invalid. Parity checking is a feature to improve the robustness of the communication.

      - Delayed data sampling

When the SPI input data arrives late at the POCI pin due to:

- Signal propagation delays

- Slow peripheral response times

- Board routing delays

The controller might sample the wrong data, i.e., it could read the previous bit instead of the current, but at the normal sampling clock edge. The solution for this is to use the CLKCTL.DSAMPLE feature, which allows you to shift the sampling point later within each clock cycle.

.

      - Clock Generation

The SPI includes a programmable bit-rate clock divider and a prescaler to generate the serial output clock (SCLK).

Bit rates supported are up to, the input clocks divided by 2. The input clock selection depends on the specific device, refer to the device data sheet and Clock Control section.

The SPI functionality can work with any of these selected inputs : BUSCLK, MFCLK and LFCLK

"Functional Clock" is the output after clock division performed according to ratio selected by the CLKDIV register. Functional clock = Selected input clock / (1 + CLKDIV)

Functional Clock x 2 (SCLK) is the output after dividing the Functional Clock by the Prescalar value. SCLK = Functional Clock / ((1 + SCR )*2)

      - FIFO Operation

## Transmit FIFO

The TX FIFO is a 16-bit-wide, 4-location-deep, first-in first-out memory buffer. The CPU writes data to the FIFO by writing the SPI Data Register TXDATA.DATA, and data is stored in the FIFO until the transmission logic reads it out.

When configured as a controller or a peripheral, parallel data is written into the TX FIFO before serial conversion and transmission to the attached peripheral or controller, respectively, through the PICO or POCI pin.

In peripheral mode, the SPI transmits data each time the controller initiates a transaction. If the TX FIFO is empty and the controller initiates a transfer, the peripheral sends undefined/random data to the transmit FIFO. The user or software is responsible for making valid data available to the FIFO as needed. The SPI can be configured to generate an interrupt or a DMA request as configured in the FIFO settings. The transmit FIFO has a TXFIFO_UNF interrupt to indicate a FIFO underflow condition.

## Receive FIFO

The RX FIFO is a 16-bit-wide, 4-location-deep, first-in-first-out memory buffer. Data received from the serial interface is stored in the buffer until read by the CPU or DMA, which accesses the read FIFO via the SPIx.RXDATA register.

When configured as a controller or peripheral, serial data is received through the POCI or PICO pin.

With the FIFO fill level trigger signals located in the STAT register (TFE, TNF, RFE, RNF), if the FIFO is full and new data is written to it without first reading data, the RXFIFO overflow event is set. The receive FIFO has an RXFULL interrupt to indicate a FIFO full condition.

      - Loopback mode

The SPI module can be placed into an internal loopback mode for diagnostic or debug work by setting the LBM bit in the CTL1 register. In loopback mode, the data from the TX FIFO can be serially transmitted into the RX FIFO. Data from the RX FIFO can be read to check whether the transmission was correct. External toggling of the IOs has no effect when the module is set to internal loopback mode.

Device boundary

SPI Module

SPI Internal Loopback

SCLK

PICO/POCI

SCLK

PICO/POCI

SCLK

POCI

PICO

CS0/1/2/3

## Figure 26-3. Internal Loop Back Mode

      - DMA Operation

The SPI provides an interface to the DMA controller with separate channels for transmit and receive. The DMA operation of the SPI is enabled through the SPI Event and DMA register. When DMA operation is enabled, the SPI asserts a DMA request on the receive or transmit channel when the associated FIFO can transfer data.

For the receive channel a transfer request is asserted whenever the amount of data in the receive FIFO is at or above the FIFO trigger level configured using the RXIFLSEL bit in IFLS register or the receive timeout has triggered, in this case the amount of data received so far will be transmitted.

For the transmit channel a transfer request is asserted whenever the transmit FIFO contains fewer characters than the FIFO trigger level configured using the TXIFLSEL bit in IFLS register. The DMA transfer requests are handled automatically by the DMA controller depending on how the DMA channel is configured.

The DMA transfers can be configured and aligned between the data width of the SPI transfers and the bus accesses width of 8/16 bits to make an efficient usage of the bus. The trigger and transfers are independent for receive and transmit.

See more information about the interrupt and event in Section 26.2.6.2 section.

      - Repeat Transfer mode

With the CTL1.REPEATTX bits the last character transmit will be repeated as defined by the register bits. A value of 0 in CTL1.REPEATTX bits will disable this mode and this function is only available in Controller mode.

The transfer will be started with writing a data into the TX Buffer. Then the data will be repeatedly sent with the given value. The behavior is identical as if the data would be written into the TX Buffer that many times as

defined by the value here. It can be used to clean a transfer or to pull a certain amount of data from a peripheral.

When REPEATTX is used it needs to be aligned with the data in the FIFO. So the below shown sequence should be used:

- Wait and check till FIFO is empty

- Setup REPEATTX

- Write to TXDATA / FIFO

- Wait till requested data has been received

      - Low Power Mode

The SPI module is located in power domain 1 (PD1) and, as such, is only active in RUN and SLEEP modes. The SPI module needs to be disabled before entering low-power mode. After waking from a low-power mode,

the SPI module must be enabled again. If the SPI module is enabled by application software, an entry into STOP or STANDBY low-power mode temporarily disables the SPI module while the device is in STOP or STANDBY mode.

### Protocol Descriptions

The protocol format mode can be selected by using CTL0.FRF register. The supported options include Motorola 3-wire, Motorola 4-wire, and Texas Instruments Synchronous.

      - Motorola SPI Frame Format

The Motorola SPI interface is a 4-wire interface where the CS signal behaves as a peripheral select. In the

3-wire mode the CS signals is not required and the module behaves as if always selected. The main feature of the Motorola SPI format is that the inactive state and phase of the SCLK signal can be programmed through the SPO and SPH bits in the SPIx.CTL0 control register.

## SPO Clock Polarity Bit

If the CTL0.SPO clock polarity control bit is clear; it produces a steady-state low value on the SCLK pin when data is not being transferred. If the CTL0.SPO bit is set; it places a steady-state high value on the SCLK pin when data is not being transferred. In 4-wire/controller mode, when CS is idle, SPICLK will be driven to its idle state. In 4-wire/peripheral mode, SCLK is expected to be in the idle state when CD is idle.

## SPH Phase-Control Bit

The CTL0.SPH phase-control bit selects the clock edge that captures data, and allows it to change state. The state of this bit has the most significant impact on the first bit transmitted, either allowing or preventing a clock transition before the first data capture edge. If the CTL0.SPH phase-control bit is clear, data is captured on the first clock edge transition. If the SPH bit is set, data is captured on the second clock edge transition. In 4-wire mode with SPH =1, the first bit is driven on the CS falling edge and captured on the first clock edge.

## Motorola SPI Frame Format with SPO = 0 and SPH = 0

Figure 26-4 shows signal sequences for Motorola SPI format with SPO = 0 and SPH = 0.

SCLK

CS

POCI

PICO

Q	MSB

MSB

4 to 16 bits

LSB	Q

LSB

Capture

Q is undeﬁned

## Figure 26-4. Motorola SPI Format With SPO = 0 and SPH = 0

In this configuration, the following occurs during idle periods:

- SCLK is forced low

- CS is forced high

- The transmit data line PICO is forced low

- When the SPI is configured as a controller, it enables the SCLK pin

- When the SPI is configured as a peripheral, it disables the SCLK pin

If the SPI is enabled and valid data is in the TX FIFO, the CS controller signal is driven low at the start of transmission which causes enabling of peripheral data onto the POCI input line of the controller. The controller PICO output pin is enabled.

In controller mode, valid controller data is transferred to the PICO pin after one functional clock. Once both the controller and peripheral data are set, the SCLK controller clock pin goes high after an additional one-half SCLK period. The data is captured on the rising edges and propagated on the falling edges of the SCLK signal.

For a single-frame transmission after all bits of the data word are transferred, the CS line is returned to its IDLE high state one SCLK period after the last bit is captured. For continuous back-to-back transmissions, the CS signal must pulse high between each frame word transfer because the peripheral-select pin freezes the data in its serial peripheral register and does not allow altering of the data if the SPH bit is clear. The controller device must raise the CS pin of the peripheral device between each data transfer to enable the serial peripheral data write. When the continuous transfer completes, the CS pin is returned to its IDLE state one SCLK period after the last bit is captured.

## Motorola SPI Frame Format with SPO = 0 and SPH = 1

Figure 26-5 shows the signal sequence for Motorola SPI format with SPO = 0 and SPH = 1.

SCLK

CS

POCI

PICO

Q	MSB

MSB

4 to 16 bits

LSB	Q

LSB

Capture

Q is undeﬁned

## Figure 26-5. Motorola SPI Frame Format With SPO = 0 and SPH = 1

In this configuration, the following occurs during idle periods:

- SCLK is forced low

- CS is forced high

- The transmit data line PICO is forced low

- When the SPI is configured as a controller, it enables the SCLK pin

- When the SPI is configured as a peripheral, it disables the SCLK pin

If SPI is enabled and valid data is present in the TX FIFO, the CS controller signal goes low at the start of transmission. The controller PICO output is enabled.

For a single-frame transfer, after all bits are transferred, the CS line is returned to its IDLE high state one SCLK period after the last bit is captured. For continuous back-to-back transfers, the CS pin is held low between successive data frame and terminates like a single-word transfer.

## Motorola SPI Frame Format with SPO = 1 and SPH = 0

Figure 26-6 shows signal sequences for Motorola SPI format with SPO = 1 and SPH = 0.

SCLK

CS

POCI

PICO

Q	MSB

MSB

4 to 16 bits

LSB	Q

LSB

Capture

Q is undeﬁned

## Figure 26-6. Motorola SPI Frame Format With SPO = 1 and SPH = 0

In this configuration, the following occurs during idle periods:

- SCLK is forced high

- CS is forced high

- The transmit data line PICO is arbitrarily forced low

- When the SPI is configured as a controller, it enables the SCLK pin

- When the SPI is configured as a peripheral, it disables the SCLK pin

If the SPI is enabled and valid data is in the TX FIFO, the SPI CS controller signal goes low at the start of transmission and transfers peripheral data onto the POCI line of the controller immediately. The controller PICO output pin is enabled.

After one Functional Clock later in controller mode, valid controller data is transferred to the PICO line. When both the controller and peripheral data have been set, the SCLK controller clock pin becomes low after one additional half SCLK period. Data is captured on the falling edges and propagated on the rising edges of the SCLK signal.

For a single-frame transmission after all bits of the data word are transferred, the CS line is returned to its IDLE high state one SCLK period after the last bit is captured. For continuous back-to-back transmissions, the CS signal must pulse high between each data word transfer as the peripheral-select pin freezes the data in its serial peripheral register and keeps it from being altered if the SPH bit is clear. The controller device must raise the CS pin of the peripheral device between each data transfer to enable the serial peripheral data write. When the

continuous transfer completes, the CS pin returns to its IDLE state one SCLK period after the last bit is captured.

## Motorola SPI Frame Format with SPO = 1 and SPH = 1

Figure 26-7shows the signal sequence for Motorola SPI format with SPO = 1 and SPH = 1.

SCLK

CS

POCI

PICO

Q	MSB

MSB

4 to 16 bits

LSB	Q

LSB

Capture

Q is undeﬁned

## Figure 26-7. Motorola SPI Frame Format With SPO = 1 and SPH = 1

In this configuration, the following occurs during idle periods:

- SCLK is forced high

- CS is forced high

- The transmit data line PICO is arbitrarily forced low

- When the SPI is configured as a controller, it enables the SCLK pin

- When the SPI is configured as a peripheral, it disables the SCLK pin

If the SPI is enabled and valid data is in the TX FIFO, the start of transmission is signified by the CS controller signal going low. The controller PICO output pin is enabled. After an additional one-half SCLK period, both controller and peripheral data are enabled onto their respective transmission lines. At the same time, SCLK is enabled with a falling-edge transition. Data is then captured on the rising edges and propagated on the falling edges of the SCLK signal.

For a single word transmission, after all bits are transferred, the CS line returns to its IDLE high state one SCLK period after the last bit is captured. For continuous back-to-back transmissions, the CS pin remains in its active low state until the final bit of the last word is captured and then returns to its IDLE state. For continuous

back-to-back transfers, the CS pin is held low between successive data words and terminates like a single-word transfer.

The serial clock (SCLK) is held inactive while the SPI is idle and SCLK transitions at the programmed frequency only during active transmission or reception of data. The IDLE state of SCLK provides a receive timeout indication that occurs when the RX FIFO still contains data after a timeout period.

      - Texas Instruments Synchronous Serial Frame Format

The SPI peripheral is compatible with Texas Instruments Synchronous Serial frame format.

Figure 26-8 shows the TI synchronous serial frame format for a single and continuous transmitted frame.

MSB

LSB

MSB

LSB

4 to 16 bits

SCLK

CS

PICO

POCI

Capture

Single Transmission Signal Sequences

LSB

MSB

LSB

MSB

LSB

MSB

LSB

MSB

4 to 16 bits

SCLK

CS

PICO

POCI

Capture

Continuous Transmission Signal Sequences

## Figure 26-8. TI Synchronous Serial Frame Format

SCLK and CS are forced low and the transmit data line PICO is put in tristate whenever the SPI is idle. When the TX FIFO contains data, CS is pulsed high for one SCLK period. The transmitted value is also transferred from the TX FIFO to the serial shift register of the transmit logic. On the next rising edge of SCLK, the MSB of the 4- to 16-bit data frame is shifted out on the PICO pin. Likewise, the MSB of the received data is shifted onto the POCI pin by the off-chip serial peripheral device. Both the SPI and the off-chip serial peripheral device then

clock each data bit into their serial shifter on each falling edge of SCLK. The received data is transferred from the serial shifter to the RX FIFO on the first rising edge of SCLK after the least significant bit (LSB) is latched.

The serial clock (SCLK) is held inactive while the SPI is idle and SCLK transitions at the programmed frequency only during active transmission or reception of data. The IDLE state of SCLK provides a receive time-out indication that occurs when the RX FIFO still contains data after a time-out period.

    - Reset Considerations

## Software Reset Considerations

A Software reset can be executed with setting the RESETASSERT together with the KEY in the RSTCTL register. An ongoing transfer will be terminated immediately and can leave the software in an undefined state. Therefore, before requesting a Reset an ongoing Transfer should be terminated.

## Hardware Reset Considerations

A hardware reset also initializes the IO configuration. This sets the IOs to a high impedance state and the data lines can float. If this is critical for the application or connected devices on the SPI interface external pull up or down resistors might be required.

### Initialization

To enable and initialize the SPI, the following steps are necessary:

- Configure the IOMUX with the appropriate GPIO pins for which the SPI signals are multiplexed to

## Note

Pull-ups can be used to avoid unnecessary toggles on the SPI pins, which can take the peripheral to a wrong state. In addition, if the SCLK signal is programmed to steady state High through the SPO bit in the CTL0 register, then software must also configure the GPIO port pin corresponding to the SCLK signal as a pull-up.

For each of the frame formats, the SPI is configured using the following steps:

- Ensure that the ENABLE bit in the CTL1 register is clear before making any configuration changes.

- Select and configure the clock prescale divisor by writing the CLKSEL and CLKDIV register.

- Select whether the SPI is a controller or peripheral:

  - For controller operations, set the CP bit in the CTL1 register.

  - For peripheral mode, clear the CP bit in the CTL1 register.

- Configure the clock divisor by writing the CLKCTL register.

- Please note that a SPI Software reset ( See section 15.3.4) is required when switching SPI protocol format.

- Configure the CTL0 and CTL1 register with based on the desired protocol, data width and other special configurations.

- Optionally configure DMA

- Enable the SPI by setting the ENABLE bit in the CTL1 register.

### Interrupt and Events Support

The SPI module contains three event publishers and no event subscribers. One event publisher (CPU_INT) manages SPI interrupt requests (IRQs) to the CPU subsystem through a static event route. The second and third event publishers (DMA_TRIG_RX, DMA_TRIG_TX) are used to setup the trigger signaling for the DMA through DMA event route.

The SPI events are summarized in Table 26-2.

## Table 26-2. SPI Events

| Event | Type | Source | Destination | Route | Configuration | Functionality |
| --- | --- | --- | --- | --- | --- | --- |
| CPU interrupt | Publisher | SPI | CPU Subsystem | Static route | CPU_INT registers | Fixed interrupt route from SPI to CPU |
| DMA trigger | Publisher | SPI | DMA | DMA event route | DMA_TRIG_RX registers | Fixed interrupt route from SPI RX to DMA |
| DMA trigger | Publisher | SPI | DMA | DMA event route | DMA_TRIG_TX registers | Fixed interrupt route from SPI TX to DMA |

      - CPU Interrupt Event Publisher (CPU_INT)

The SPI module provides 9 interrupt sources that can source a CPU interrupt event. Table 26-3 lists the CPU interrupt events from the SPI in order of decreasing priority.

## Table 26-3. SPI CPU_INT Trigger Condition

| IIDX STAT | Name | Description |
| --- | --- | --- |
| 0x01 | RXFIFO_OVF | RXFIFO overflow event. This interrupt is set if an RX FIFO overflow has been detected. |
| 0x02 | PER | Parity error event. This bit if a Parity error has been detected. |
| 0x03 | RTOUT | Peripheral receive timeout event. When in peripheral mode and not receiving data for the CTL1.RXTIMEOUT selected number of functional clock cycles. |
| 0x04 | RX | Receive FIFO event. This interrupt is set if the selected receive FIFO level has been reached. |
| 0x05 | TX | Transmit FIFO event. This interrupt is set if the selected transmit FIFO level has been reached. |
| 0x06 | TXEMPTY | Transmit FIFO empty interrupt. This is set if all data in the transmit FIFO have been shifted out. |
| 0x07 | IDLE | SPI Idle. SPI has done finished transfers and changed into IDLE mode. This bit is set when STAT.BUSY bit goes low. |
| 0x08 | DMA_DONE1_RX | This interrupt is set if the RX DMA channel sends the DONE signal. |
| 0x09 | DMA_DONE1_TX | This interrupt is set if the TX DMA channel sends the DONE signal. |

The CPU interrupt event configuration is managed with the CPU_INT event management registers. See Section

7.2.5 for guidance on configuring the Event registers.

      - DMA Trigger Publisher (DMA_TRIG_RX, DMA_TRIG_TX)

DMA_TRIG_RX and DMA_TRIG_TX registers are used to setup the trigger signaling for the DMA. This can be setup in a flexible way to trigger the DMA for receive or transmit events with the trigger conditions in Table 26-4 and Table 26-5.

DMA_TRIG_RX is used for triggering the DMA to do a receive data transfer and DMA_TRIG_TX is used for triggering the DMA to do a transmit data transfer.

## Table 26-4. SPI DMA_TRIG_RX DMA Trigger Condition

| IIDX STAT | Name | Description |
| --- | --- | --- |
| 0x03 | RTOUT | Peripheral receive timeout event. When in peripheral mode and not receiving data for the CTL1.RXTIMEOUT selected number of functional clock cycles. |
| 0x04 | RX | Receive FIFO event. This interrupt is set if the selected receive FIFO level has been reached. |

Table 26-5. SPI DMA_TRIG_TX DMA Trigger Condition

| IIDX STAT | Name | Description |
| --- | --- | --- |
| 0x05 | TX | Transmit FIFO event. This interrupt is set if the selected transmit FIFO level has been reached. |

The DMA trigger event configuration is managed with the DMA_TRIG_RX and DMA_TRIG_TX event management registers. See Section 7.2.5 for guidance on configuring the Event registers and Section 7.1.3.2 for on how DMA trigger event works.

### Emulation Modes

The module behavior while the device is in debug mode is controlled by the FREE and SOFT bits in PDBGCTL register.

When the device is in debug mode and set into halt mode below behavior can be configured.

Table 26-6. Debug Mode Peripheral Behavior

| PDBGCTL.FREE | PDBGCTL.SOFT | Function |
| --- | --- | --- |
| 1 | x | Modules continues operation |
| 0 | 0 | Module stops immediately |
| 0 | 1 | Module stops after the next transfer has been finished |